Intel® Celeron® Quad Core processor

Fanless Industrial Computer

IBOX-1026

Features：

◇ All aluminum structure，fanless design；

◇Onboard Intel Celeron J4125 Quad Core processor；

◇ Support 2*Intel i211-AT 1000M LAN；

◇ Support 1*M.2 Slot,1*Mini PCIE Slot；

◇ Support WIFI/3G/4G (Three choose one)

◇ Support 6*COM,COM1 Support RS485；

◇ Support OS：

| Specification |  |
| --- | --- |
| Model | IBOX-1026 |
| Color | Black（Color can be customized） |
| Material | High-quality all aluminum，surface anodic oxidation |
| Processor | Onboard Intel Celeron J4125（Quad core quad threading，4MB Cache，2.0GHz，turbo 2.7GHz） |
| CPU Architecture | Gemini Lake Refresh |
| RAM | Support 1*DDR4 2400MHz SODIMM Memory Slot，Max. Support 8GB |
| BIOS | AMI UEFI BIOS |
| Display Chip | Integrated Intel UHD Graphics 600 |
| Display Port | 1*HDMI,1*VGA（Support synchronous and asynchronous dual display,Max Resolution support 4096x2160 @ 30Hz） |
| Front I/O | 4*USB2.0，1*Power Switch,1*COM |
| Rear I/O | 1*DC,1*VGA,1*HDMI, 2*USB3.0， 2*1000M LAN, 1*MIC,1*SPK |
|  | 5*COM |
| Expansion Slot | Support 1*M.2 2280 M-key Slot（Support SATA Protocol） |
|  | Support1*Mini PCIE Slot（4G/WIFI/BT Optional，Embedded SIM slot） |
|  | Optional Ports：GPIO、4*USB2.0、PS/2 、LPT |
| Network | 2*Intel I211-AT 10/100/1000M Ethernet LAN |
| Storage | 1*M.2 2280 Slot，1*2.5’’ HDD |
| Input Voltage | DC 12V Input |
| Other Function | Auto power on when there is electricity、Wake on LAN、PXE boot |
| Work Temp | 0℃ - +50℃（Commercial HDD），-20℃ - +60℃（Industrial SSD），Surface air flow |
| Work Humidity | 0% - 95% non-condensing |
| Certificates | CE、 CCC、 FCC Class A、 ROHS |
| Dimension | 234 * 200 * 54 mm |
| Net Weight | 2.0kg |
| Application | Industrial Automation, Medical, Logistics Transportation, Warehouse, Electronic Education etc. |

IBOX-1026 Drawing

| Accessories List |  |  |
| --- | --- | --- |
| Name | Quantity | Specification |
| Power Adapter | 1 | Power Cord 12V 4A/5A |
| Power Cord | 1 | Optional (China, US, EU, UK etc.) |

| Order Information |  |  |  |
| --- | --- | --- | --- |
| Configuration 1 | IBOX-1026 | Intel Celeron J4125 | Standard Ports |